```

EMERGENCY RESPONSE PLAN

For:	CAMBRIDGE CENTRE FOR ADVANCED RESEARCH AND EDUCATION IN SINGAPORE LTD.

Venue:	1 CREATE Way

#07-06/07/08 Research Wing

Singapore 138602

NOTE: This Emergency Response Plan (ERP) has been prepared by Cambridge Centre for Advanced Research and Education in Singapore Ltd. (CARES) for its laboratory facility at the above address in accordance with Clause 27 of Singapore’s Fire Safety (Petroleum and Flammable Materials) Regulations (2008) and the Emergency Response Plan guidelines (2014) of the Singapore Civil Defence Force (SCDF).

It is CARES’s policy that all personnel working within CARES shall be familiar with this ERP. Each revision of the ERP shall be submitted to SCDF for approval and each submitted ERP shall be valid for 12 months only. Re-submission of the ERP on an annual basis is required even if no changes have been made.

This ERP is applicable to CARES’s laboratory facility at the above address ONLY.

NOTE: This Emergency Response Plan shall be handed over to SCDF responders during an emergency.

2020

VERSION 1.8

# TABLE OF CONTENTS

TABLE OF CONTENTS	2

TABLE OF RECORDS OF EMERGENCY PLAN REVISION	3

TABLE OF RECORDS OF EMERGENCY EXERCISES CONDUCTED	4

1	AIM	6

2	SITUATION	6

3	EXECUTION	14

4	SERVICE SUPPORT	24

5	COMMAND AND SIGNAL	26

6	PLAN REVIEW & MAINTENANCE	26

ANNEX A – CREATE BUILDING SITE PLAN	28

ANNEX B – CREATE BUILDING RESAERCH WING LEVEL 7 PLAN	30

ANNEX B-2 – LOCATION OF ASSEMBLY AREA	31

ANNEX C – CHEMICAL INVENTORY	32

ANNEX D - PREMISES EMERGENCY DATA	77

ANNEX E – OTHER PLANS	83

ANNEX F – ROLES AND RESPONSIBILITIES	84

ANNEX G – DETAILED GROUPING AND TASKS	90

ANNEX H – EMERGENCY SPILL CONTAINMENT PROTOCOLS	91

ANNEX I – CERT STRUCTURE	93

ANNEX J – EMERGENCY RESPONSE FLOWCHART	94

# TABLE OF RECORDS OF EMERGENCY PLAN REVISION

| S/N | Plan Version | Date of Approval* | Name and Appointment of Approving Personnel** | Signature of Approving Personnel | Remarks |
| --- | --- | --- | --- | --- | --- |
| 01 | 1.0 | 07/03/16 | Prof. Markus Kraft |  |  |
| 03 | 1.3 | 31/05/16 | Hugo Gerald Schmidt |  | Following board meeting, suggestion was made to amend section 3.2.1. On revision, it has been concluded that section is in accord with good ERP practices |
| 04 | 1.4 | 28/02/16 | Hugo Gerald Schmidt |  | Instructions on how to deal with contractors |
| 06 | 1.6 | 15/03/17 | Hugo Gerald Schmidt |  | Update of SIC responsibilities and contractor evacuation |
| 07 | 1.7 | 22/11/17 | Hugo Gerald Schmidt |  | Update of Annex I |
|  |  | 16/5/18 | CARES Governing Board |  | Oversight and endorsement of changes to date |
| 08 | 1.8 | 21/10/18 | Hugo Gerald Schmidt |  | Update of ERT / First Aider Personnel |
| 09 | 1.9 | 12/09/19 | Hugo Gerald Schmidt |  | Update of ERT / First aider personnel and waste disposal service |
| 10 | 1.10 | 21/04/20 | Hugo Gerald Schmidt |  | Updated to include PIPS and eC02; revise C4T to fit Phase 2. |

*	All dates in DD/MM/YY

** 	The Emergency Response Plan should be endorsed and approved by CARES’s senior management.

*** 	The updated Emergency Response Plan shall only be issued after it has been approved by SCDF via online submission (http://www.scdf.gov.sg/content/scdf_internet/en/building-professionals/e-services.html)

# TABLE OF RECORDS OF EMERGENCY EXERCISES CONDUCTED

| S/N | Date of Exercise | Name and Appointment of Conducting Personnel | Signature of Conducting Personnel | Comments |
| --- | --- | --- | --- | --- |
| 01 | 15/08/2016 | Hugo Gerald Schmidt |  | Good attendance and performance |
| 02 | 02/05/2017 | Hugo Gerald Schmidt |  | Good attendance. Some needed additional instruction to perform their duties, but the lessons were absorbed |
| 03 | 27/03/2018 | Hugo Gerald Schmidt |  | Admirable performance. The lessons from previous session were absorbed well. |
| 04 | 11/06/2019 | Hugo Gerald Schmidt |  | Good performance – the sole failure to perform correctly was corrected. |

Note:

Emergency Exercise shall be conducted at least once a year in order to validate this Emergency Response Plan. Exercise shall include familiarity with emergency evacuation route, location of emergency evacuation assembly area, spill control protocols, documents upkeep, etc.

# Glossary of Terms

ABC Fire Extinguisher: Powder based fire extinguisher that can be used on combustible materials (e.g. wood, paper etc.), liquid flammables (e.g. petroleum products) and electrical fires

CERT: Company Emergency Response Team

ERP: Emergency Response Plan

Fire, Class A:  Combustible materials (wood, paper, refuse etc.)

Fire, Class B:  Flammable liquids and flammable gas

Fire, Class C: Gas Fire

HAZMAT:  Hazardous Materials

IPP:  In Place Protection

IRP: Interdisciplinary Research Project

LEL:  Lower Explosive Limit, the lowest concentration of a gas or vapour in air capable of exploding in the presence of an ignition source (heat, fire, spark)

MAQ:  Maximum Allowable Quantities

MOM:  Ministry of Manpower

NEA: National Environment Agency

NRF: National Research Foundation

PFM: Petroleum & Flammable Materials

PPE:  Personal Protective Equipment

SCDF: Singapore Civil Defence Force

Scheduled Premise:  CARES’s R&D laboratory on level 7 of CREATE Way

SDS: Safety Data Sheet

EMERGENCY RESPONSE PLAN

# 1	AIM

This emergency response plan details the operational actions that designated staff of Cambridge Centre for Advanced Research and Education in Singapore Ltd. (“CARES”) shall take during an occurrence of a fire or hazmat emergency with the aim of minimising injury to personnel and damage to property and the environment.

It has been specifically developed to address potential emergency situations arising from the use of hazardous chemicals, gases and substances and scheduled petroleum and flammable materials (PFM) within CARES’s R&D laboratory at 1 CREATE Way, #07-06/07/08 Research Wing, Singapore 138602 (Scheduled Premise).

Hazardous chemicals/substances are those defined in the Singapore’s Workplace Safety and Health Act and/or listed in the Environmental Protection and Management Act and the Environmental Protection and Management (Hazardous Substances) Regulations. Petroleum and Flammable Materials (PFM) are flammable chemicals and gases listed in the Second Schedule of the Fire Safety (Petroleum & Flammable Materials) Regulation 2008. Explosive precursors are chemicals defined in Singapore’s regulated explosive precursor list maintained by the Singapore Police Force.  Sources of Ionising Radiation are defined in the Singapore Radiation Protection Act.

This Plan shall be read in conjunction with the Emergency Response Plan implemented by the National Research Foundation (NRF) for the CREATE campus.

# 2	SITUATION

This section details CARES’s background information which includes its nature of business, location, neighbours and its inherent risks and hazards.

## INTRODUCTION

Cambridge Centre for Advanced Research and Education in Singapore Ltd. (“CARES”) is a private limited company incorporated in Singapore. It is a wholly owned subsidiary of University of Cambridge in the United Kingdom. The company was created to administer the CREATE programme of the University of Cambridge in Singapore (funded by the National Research Foundation), which hosts a number of research collaborations with local academic institutions and industrial partners.

The CARES laboratory at the CREATE campus brings together researchers from multiple disciplines to undertake research projects in materials sciences, including the development of energy efficient chemical processes with the aim of reducing carbon footprint and energy demand.

The research centre focuses on 6 areas:

IRP 1: Multi-Scale Studies of Catalytic and Adsorption Technologies (MUSCAT)

IRP 2: Electrochemical Multi-Scale Science, Engineering and Technology (EMSET)

IRP 3: Carbon Abatement in the Petroleum Refining Industry: A Control and Optimisation Research Network (CAPRICORN)

IRP 4: Better, Cleaner Heat Usage

eCO2EP: conversion of Carbon Dioxide to value added products such as ethylene and 1-propanol

PIPS: Pharma Innovation Programme Singapore.  New approaches to process development

Experiments that will be performed involves chemical reactions from room temperature up to 1200°C and atmospheric pressure to 200 bar, naked flames, nano-particles, fabrication and testing of micro/nano-devices and electrodes for batteries. Varying quantities of hazardous and flammable chemicals, including toxic and flammable gases, are used in support of the various research activities. The respective quantities are denoted in Annex C of this ERP.

## 2.2	AREA OF OPERATIONS

### 2.2.1	Location of Premises and its main access road

CARES’s R&D laboratory is located at 1 CREATE Way, #07-06/07/08 Research Wing, Singapore 138602. The main vehicle access to the Research Building is via CREATE Way. Please refer to Annex A for the Site Plan.

### 2.2.2	General Description of the surroundings and neighboring premises

The Research Wing building within the CREATE Campus is a 7-storey building with 846 square metres space on level 7.  Adjacent to the space are the lower roofs of the Innovation and Enterprise wings of the CREATE Campus. The CREATE campus is administered by the National Research Foundation (NRF), which also oversees the other research institutes in its CREATE programme.

ENTERPRISE WING

LEVEL 7

RESEARCH WING

INNOVATION WING

CARES Lab

### 2.2.3    Layout of premises

The detailed layout plan of the Scheduled Premise is shown in Annex B with the locations of the flammable chemical storage cabinets and flammable gas cylinder storage cabinets highlighted.

The list and maximum quantities of petroleum & flammable materials (PFM) and hazardous materials (HAZMAT) stored within the Scheduled Premise is shown in Annex C. Updated inventory of the different chemicals is maintained by the designated personnel within CARES.

The Safety Data Sheets (SDS) of the various petroleum & flammable materials and hazardous materials used / stored at the Scheduled Premise are maintained by CARES and will be handed over to the SCDF responders in times of emergencies.

## 2.3	SPECIAL HAZARD & RISK ASSESSMENT

The main hazards within the R&D laboratory are summarized in the Premise Emergency Data Sheet (Annex D). Specifically, these hazards include:

- Pressure vessels

- Hazards

A number of chemical reactors within the Company will be operated at elevated pressure (> 0.5barg) and become pressure vessels. In the presence of engineering defects, damage or reaction run away, pressure vessels may rupture, accompanied by an uncontrolled release of substances and energy stored within, especially when the pressurised substances are compressible, e.g. gases. Rupture of pressure vessels will lead to damage of equipment, personnel injuries, gas leakage and chemical spills.

- Emergency / Mitigation Measures

  - Pressure run-away

Pressure relief valves are installed on all professionally certified pressurised reactors. In case all pressure relief valves fail close in the event of a pressure run-away, the operator must close all gas and electrical supplies in the first instance, leave the room unoccupied and locked, inform lab manager, and alert all occupiers of the situation. Wait for 24 hours before re-entrance to the room. Follow the evacuation procedure (see sections below for evacuation procedure) if an evacuation alarm is triggered.

In case the pressure runaway takes place during unattended running, a rupture is immanent. If the rupture triggers an evacuation alarm, then the evacuation procedure should be followed. If the rupture does not trigger an evacuation alarm, the operation should report the incident to the lab manager in the first instance.

b. Mechanical defects

Mechanical defects of the pressure vessel may result in a rupture prior to any pressure relief or alert. If the rupture causes personnel injuries, the first aiders on duty should be informed immediately and any first aid actions should take place if it is safe to do so. Call 995 or 999 if the injury appears severe. Shut off all gas and electrical supplies if it is safe to do so, evacuate and lock the room with the pressure vessel. Report the incident to the lab manager.

- Safety Control Measures

All pressure vessels should be tested and certified by qualified professional engineers. During operation, pressure vessels should be isolated inside the high pressure hutches with all doors kept shut. All emergency shut down procedures should be clearly displayed near the pressure vessels. All operator of pressure vessels should be trained to qualify for the permit-to-work. Modification of commercial pressure vessels should only be performed by those with a professional certificate and any modification must be reported to and recorded by the lab manager. All pressure relief valves should be checked on an annual basis. Prior to every operation, the pressure vessel must be leak checked and tested at 0.5 barg using an inert gas.

- Fire / Chemical Exposure from spilled chemicals

- Hazards

A number of chemicals used within the Company are highly flammable chemical classified as a CLASS 3 flammable liquid with low flash points. They are reactive with oxidising agents, acids and alkalis. These chemicals may ignite or explode in the presence of open flames, heat or sparks and in the presence of oxidising materials. The chemical may burn, potentially with near invisible flame. Other chemicals may be corrosive or toxic and pose exposure hazards when inhaled, ingested or in contact with skin.

- Emergency / Mitigation Measures

In the event of small fire, the Type ABC fire extinguisher found within the lab should be used to fight Class A and B fires. Alternatively, the carbon dioxide (CO2) fire extinguisher may be used to extinguish Class B fires. The respective researchers shall be responsible for performing risk assessment to ascertain the proper type of fire extinguisher to use and to ensure that the proper type of fire extinguisher is available before use of any hazardous chemical. Specifically, ABC dry chemical extinguishing agents are incompatible with strong alkalis (bases), magnesium, strong oxidisers such as calcium hypochlorite and isocyanuric acids. Furthermore, the CO2 extinguishers do not work well on Class A fires.

Small spills should be diluted with water and mopped up, or absorbed using a chemical sorbent. The sorbents should be placed in a chemical waste container for off-site disposal by a licensed toxic industrial waste collector.

For larger spill, personnel shall first don the appropriate respirator prior to the cleanup to minimise chemical exposure. Respirators shall be fit tested prior to use. The correct type of chemical resistant personal protective equipment (PPEs) shall also be used (e.g. appropriate type of gloves). All chemical waste shall be collected for off-site disposal by a licensed toxic industrial waste collector.

- Safety Control Measures

- Water sprinklers are installed within the Scheduled Premise to fight all fires. These sprinklers shall be maintained on a periodical basis. Researchers should note that water sprinklers could be automatically triggered at elevated temperature (68C) and measures should be taken to prevent water reactive chemicals or reactions from coming into direct contact with the sprinkler water.

- Flammable chemicals shall be stored within the flammables storage cabinet when not in use.

- The company shall not store excessive quantity of hazardous chemicals within the Scheduled Premise. Instead, it shall arrange for more frequent delivery with chemical vendors.

- The bottle size of all hazardous chemicals shall be 18 liters.

- Care shall be taken when using acids or alkalis in the presence of flammable chemicals to minimise chance of undesired reaction

- All new experiments shall be risk assessed and appropriate control measures put in place prior to execution. Researchers shall always verify if additional safety gears not already provided for are required for the safe handling of new hazardous materials and experiments.

- Hazardous chemicals shall only be handled within ventilated outlets (e.g. fume hoods, canopy hoods or local fume extractors).

- Appropriate personal protective equipment shall be used when handling hazardous chemicals as recommended in the respective chemical’s SDS.

- Compressed gas cylinders

The compressed gas cylinders used at the lab include:

- hydrogen (H2),

- methane (CH4),

- acetylene (C2H2).

- ethylene (C2H4),

- ethane (C2H6),

- propane (C3H8),

- carbon monoxide (CO)

- carbon dioxide (CO2)

- oxygen (O2)

- hydrogen sulfide (100ppm H2S in N2)

- helium (He)

- argon (Ar)

- nitrogen (N2)

- gas mixtures containing one of the hazardous gases above in inert carrier gas

- Hazards

A variety of compressed gas cylinders are used within the lab facility. The hazards associated with these gases may be broadly classified into 5 main areas:

- High pressure – all compressed gases are stored in cylinders at high pressure. Furthermore, they are delivered to the various points of use at varying pressures ranging from 3 bars to 50 bars.

- Flammable – a number of compressed combustible gases, including hydrogen (H2), methane (CH4), acetylene (C2H2). ethylene (C2H4),  ethane (C2H6), propane (C3H8), carbon monoxide (CO) and gas mixtures containing these gases may be highly flammable and can readily form explosive mixtures with air over a very wide concentration range. When involved in a fire, these gases may also decompose to produce toxic gases, including carbon monoxide and carbon dioxide.

- Toxic Gases – Carbon monoxide (CO) and hydrogen sulfide (H2S) are toxic gases handled within the facility. Inhalation of high levels of these gases can lead to toxic effects.

- Oxidising gas – The oxygen gas used is an odorless, colourless, nonflammable gas. It is an oxidising gas and vigorously accelerates combustion. Breathing air containing high concentration of oxygen or oxygen at higher pressure may also cause breathing difficulty or lung damage.

- Corrosive gas – Hydrogen sulfide gas is a corrosive gas and may cause corrosion to some metals when wet or at elevated temperature. Hydrogen sulfide can also attack a number of plastics and care should be taken to select proper tubings and containers when handling this gas.

- Asphyxiants – All gases noted above as well as the other inert gases used in the lab [helium (He), argon (Ar) and nitrogen (N2)] are asphyxiants and can cause suffocation when concentrations are sufficient to reduce oxygen levels below 19.5%.

- Emergency / Mitigation Measures

During an emergency, the preferred mitigation approach for compressed gases is to minimise contact with fugitive gas emission, thereby reducing exposure to the ill-effects of these gases.

This can be achieved by both a) increasing lab ventilation to discharge the fugitive gases from the lab and b) terminating the supply of the compressed gases within the lab.

Ventilation in the lab is organised such that there are at least 8 changes of fresh air into the lab per hour. Higher air change rates may be achieved when the fume hood sashes are drawn up, when the exhaust rates on variable speed fume extractors or canopy hoods are increased or when a fume hood exhaust purge button is pressed. Ventilation may also be temporarily elevated by increasing the exhaust set point through the building management software system.

Furthermore, all gas distribution lines are installed with solenoid valves near to the gas cylinders that would automatically close and terminate ALL gas supplies to the lab facility when:

- The lab exhaust system is non-operational (i.e. exhaust fans are not running);

- The respective solenoid valve control buttons at the gas system control panel outside the lab facility (along the main corridor) are pressed;

- A gas leak detector has detected elevated level of the hazardous gas or reduced level of oxygen (during stage 2 detector alarm);

- All hazardous gases are carried in coaxial piping system with the inner pipe carrying the hazardous gases and the outer pipe filled with nitrogen gas to 3 bar. A rupture or leak in the inner pipe would result in elevated pressure in the interstitial space. When this happens, an alarm would sound and the solenoid valves to all gas cylinders would close; or

- One of the gas emergency panic buttons around the lab is pressed.

The respective researchers shall be responsible for performing risk assessment to ascertain the appropriate emergency measures as well as the proper type of fire extinguisher to use and to ensure that the proper type of fire extinguisher is available before use of any hazardous chemical.

The Singapore Standards for Use and Maintenance of Portable Fire Extinguishers (SS578) discouraged the fighting of Class C (flammable gas) & D (flammable metals) fires by untrained individuals. The fire extinguishers found within the facility are not rated for use on Class C or Class D fires. The Singapore Standards recommends that flammable gas fires be extinguished by closing the gas valves or plugging the gas leak.

All personnel shall be evacuated from the lab during a gas emergency. No one shall re-enter the premise until the concentration of fugitive gas within the space is deemed too low to pose a health hazard. Prior to re-entry, the space shall first be inspected by a designated and trained CARES personnel using a portable multi gas detector and wearing a fitted self-contained breathing apparatus.

If a leak has been detected, the source of the leak should be identified and rectified before the specific gas supply is re-activated.

- Safety Control Measures

- All flammable gas cylinders shall be stored within 90 minutes rated gas cylinder storage cabinets. These cabinets shall be kept closed at all times and care shall be taken when opening any of the cabinet doors during a gas emergency.

- All gas cylinders shall be secured using brackets.

- The lab shall limit the quantities of gas cylinders stored within the Scheduled Premise to no more than the Maximum Allowable Quantities (MAQ) as denoted in Annex C.

- Existing gas piping shall not be used for delivery of any other gas without prior detailed risk assessment and approval from CARES Director. The entire gas piping shall be re-labelled to denote the new gas prior to delivery.

- The gas cylinder regulator and gas delivery piping and tubing shall be checked on a periodical basis against defects and leaks.

- The integrity of the coaxial piping system shall be checked periodically - the pressure of the nitrogen gas in the interstitial space shall be verified and proper functioning of the high/low pressure gauges checked periodically.

- The Lower Explosive Limit (LEL), carbon monoxide, carbon dioxide, oxygen and hydrogen sulfide gas detectors shall be periodically inspected and calibrated to ensure their continual performance.

- The proper functioning of the solenoid valves shall be checked periodically.

- The lab facility is the property of the National Research Foundation (NRF). No gas line modification work is permitted without the prior written consent of NRF. The gas system design has also been validated by a Qualified Person and no modification of the design is permitted without prior written consent of the Qualified Person/Professional Engineer and new design plan submission to the Singapore Civil Defence Force (SCDF).

- Additional safety features in the gas piping system include control valves and these valves shall be physically checked and pressure tested on a periodical basis to ensure proper functioning:

  - Piping material is high quality SS316 and orbital welded to ensure high weld integrity.

  - Flame arrestors are installed at both the cylinder and point of use ends of all flammable gas lines.

  - Check valves are installed to ensure unidirectional flow of all gases from the cylinder to the points of use.

  - Excess flow valves are installed at each cylinder end of the gas line to limit maximum gas flow rate and to cut gas flow during a major gas line rupture.

  - Pressure release valves are installed to prevent unwarranted pressure build-up along the delivery line.

Ball valves are installed to control direction of gas delivery.

- Ionising Radiation Sources

Sources of ionising radiation used in the lab include:

Americanum-241

  - Hazards

    - Radiation poisoning.  Though unlikely to be harmfuI if ingested, an alpha particle source could lead to radiation poisoning

    - Teratogenic effects.  Ingestion of alpha particle emitter could lead to spontaneous abortion or foetal abnormalities.

    - Theft or sabotage.  Radioactive materials are sought by terrorist or extremist factions for use in terrorist activities.

  - Emergency / Mitigation measure

The alpha-particle emitter is solid state and so can be more easily contained than comparable gaseous state emitters.  In the event of a breach of the containment of the radiation emitter, the source will immediately be confined in a vessel capable of containing of alpha particles.  All staff that have been exposed will be immediately referred to medical checkup to ensure their health. A verbal report is to be made to the NEA chief within 24 hours and a written report within 10 days.

Breaches of containment are detected by either the daily inspection of the source and storage area with a Geiger Radiation Counter or by the wearable dosimeters that all radiation workers must wear while engaging in radiation work.

  - Safety Control Measures

    - The radiation source is kept in a locked cupboard, and examined daily for loss or removal.  The cabinet is subject to 24 hour CCTV monitoring.  Any absence of the radiation source results in an immediate report to the Singapore Police Force, and a review of the CCTV footage.  The CCTV footage is stored in a separate building to the radiation source, making tampering very difficult.

    - The storage location and the radiation source are tested daily with a Geiger radiation counter for leaks.

    - All laboratories and work areas where radiation work is being carried out are clearly marked as such.

# 3	EXECUTION

This section highlights all main credible scenarios (such as fire or Hazmat incident) and the necessary actions to be taken by the company during the emergency.

## 3.1	CONCEPT OF OPERATIONS

The emergency operation to be conducted in phases is as follows:

| Phase |  | Actions |
| --- | --- | --- |
| I |  | To notify building Fire Command Centre and thereafter SCDF and SPF for all emergency and mass casualty (10 or more casualties) incidents. To notify other related agencies (NEA, MOM, etc) and surrounding companies if necessary. |
| II | a | Company personnel to conduct evacuation operations. Receives confirmation of evacuation from contractors. |
|  | b | Company personnel to initiate emergency actions to mitigate or contain the emergency and coordinate with SCDF personnel upon SCDF’s arrival. |
| III |  | To clean up / decontaminate and/or drain leaking gas piping containing hazardous gases and ventilate area prior to resuming normal operations. For gas emergency, the safety committee shall convene an on-site meeting to discuss and execute the most appropriate recovery measure prior to allowing the rest of staff back into the lab. |

Note:

Phase II (a) and (b) should be conducted simultaneously or as directed by the Site Controller / Site Incident Controller and representative/s from the Building Fire Command Centre, including the Building Fire Safety Manager.

## 3.2	EMERGENCY ACTIONS TO BE TAKEN

### 3.2.1	(Phase I) Procedure to notify SCDF, SPF. To notify other related agencies and surrounding companies

During office hour, any personnel who discover a fire or emergency situation shall immediately raise the Fire Alarm by activating the nearest Fire Alarm “Break Glass” Call Point located both within and outside the lab. The Site Controller shall be informed at the earliest possible time.

Note that the Gas Emergency Buttons located at various lab exits would ONLY trigger the closure of the solenoid valves to all piped gases into the lab. They would also activate the Gas Emergency Alarm and buzzer within the lab but this would not activate the building Fire Alarm.

Additional room specific gas emergency buttons that would only stop gas delivery to the respective rooms are located at the far end of Lab 1. These room specific buttons would not trigger the Gas Emergency Alarm.

The following table summarises the actions to be taken during the different emergency events:

| Type of emergency | Gas Emergency Alarm | Fire Alarm |
| --- | --- | --- |
| Gas Emergency within lab | Activate Gas Emergency button. Gas solenoid valves for all gases will close. Gas Evacuation Alarm (RED alarm) and buzzer will be triggered. Evacuate lab and report to Assembly Area. Contact Gas Safety contractor. | Assess if Fire Alarm shall also be activated. When in doubt, activate Fire Alarm via the fire alarm call point to alert the building Fire Command Centre. Building Fire Safety Manager shall decide if rest of building shall also be evacuated. |
| Gas leak within lab | Stage 1 gas alarm (ORANGE Alarm) and buzzer will be automatically triggered when low level of fugitive gas is detected. The designated CARES personnel shall investigate the source of the leak. No evacuation is required. Gas solenoid valves for all gases will be automatically closed when Stage 2 gas detector alarm is triggered or when high pressure (>3.5 bar) is detected within the coaxial piping interstitial space. Gas Evacuation Alarm (RED Alarm) and buzzer will be automatically triggered. Evacuate lab and report to Assembly Area. Contact Gas Safety Contractor | The Fire Alarm will not be activated but a gas leak alarm on the gas detector panel at the building Fire Command Centre will be triggered. The building Fire Safety Manager shall decide if rest of building is to be evacuated. |
| Fire Emergency within lab | Activate Gas Emergency button. Gas solenoid valves for all gases will close. Gas Evacuation Alarm and buzzer will be triggered. Evacuate lab and report to Assembly Area | Activate Fire Alarm. Evacuate lab and report to Assembly Area. Building Fire Safety Manager shall decide if rest of building shall also be evacuated. |
| Large solid or liquid biological or chemical spill within lab | Assess if Gas Emergency button shall be pressed. When in doubt, press Gas Emergency button to shut off all gases. Evacuate lab and report to Assembly Area | Activate Fire Alarm. Evacuate lab and report to Assembly Area. Building Fire Safety Manager shall decide if rest of building shall also be evacuated. |

Alternatively, if one hears the Fire Alarm and know that it has NOT been activated by anyone from within the facility, he/she shall first check the premise to see if the emergency originates from within. The Fire Alarm signal may be automatically raised when there is an activation of the water sprinkler system.

If an emergency within the lab is confirmed, the first person shall report the emergency to the building’s Fire Command Center and note the following:

- Location of fire / LARGE biological or chemical spill / Gas Emergency

- Size of fire / type of chemical / biological agent spilled / Gas Emergency details

- Injury to personnel, if known

- Extent of the affected area

| Key Contact Directory | Tel |
| --- | --- |
| NUS Fire Command Centre | 6601 2630 (Main) 6684 0616 (Backup) |
| Building Management Office | 6601 4011 6601 4012 |
| Security Center @ Level 2 Tower Block | 6601 2633 |
| Safetech Gas Safety Contractor | 9690 3885 (main) 8383 6657 (backup) |

Once the Fire Alarm has been triggered, the Fire Alarm bell on all floors will ring for about one minute before they are isolated. An indication will also be displayed on the building’s fire alarm panel at level 7 and at the Fire Command Centre in the basement to indicate the location where the Fire Alarm has been activated. Note that the lab Gas Emergency Alarm is not interlinked with the building Fire Alarm. The activation of one alarm will not automatically trigger the other alarm.

The first person shall also assess if the Gas Emergency Button shall be pressed to terminate supply of all gases within the lab. When in doubt, the Gas Emergency Alarm shall be activated to reduce risk associated with compressed and hazardous gases. The Gas Emergency Alarm within the lab will then be activated and all gas solenoid valves closed.

All lifts will return to the ground floor with doors opened and the building’s security personnel on duty will broadcast a general alert announcement via the building’s Public Address System. This shall be known as the 1st stage alarm.

The Building Fire Safety Manager shall serve the role of liaison officer for the CREATE building to communicate with SCDF’s Ground Commander. CARES appointed Site Controller shall represent CARES and provide company-specific information and render assistance, when required, to the Building Fire Safety Manager and SCDF Ground Commander.

The Building Fire Safety Manager shall assess and decide if other tenants in neighboring buildings shall also be evacuated. If so, the Building Fire Safety Manager shall coordinate the evacuation.

The number of staff and researchers within the Scheduled Premise during operating hours (9:00am to 6:00pm) is 36. However, given that this is a research lab facility, we expect researchers to work outside of the normal operating hours on a frequent basis. The number of researchers during the normal operating hours may however be less.

The Company Site Controller shall notify the Pollution Control Department of the National Environment Agency (NEA) at Tel: 1800-CALL NEA (1800-2255 632) should a large chemical spill situation with potential environment pollution concerns occur.

The Company Site Controller shall also notify the Commissioner for Workplace Safety and Health at the Ministry of Manpower (MOM) immediately at Tel +65 6317 1111 should an incident result in fatality or if the incident is classified as a Dangerous Occurrence as defined by MOM. All work accidents causing death or injury, incidence of occupational disease and dangerous occurrence shall also be reported to MOM via the MOM’s i-report portal (www.mom.gov.sg/ireport).

If medical attention is required and if the casualty is conscious and is able to move on his/her own, the Site Controller may instruct an employee to deliver the casualty to the nearest hospital. The nearest hospital is the National University Hospital at 5 Lower Kent Ridge Road, Singapore 119074 (Tel: 6779 5555) – located less than 5 minutes away by car. Else, an ambulance should be summoned by dialling 995.

In the event of an emergency during after office hour and when no CARES researcher is present, the security personnel on duty shall inform the Fire Command Centre immediately. The latter shall then notify SCDF of the emergency. The Fire Command Centre will also contact CARES representative via CARES’s duty phone to obtain company-specific information if the fire or emergency is thought to have originated from within the Scheduled Premise. CARES’s duty personnel shall assess the need to return to the facility without delay to investigate the emergency.

### 3.2.2	(Phase II a) Procedure for emergency evacuation

On hearing the 1st stage Fire Alarm, all staff shall lock up their respective important files and shut down the various machineries and lab equipment. All chemicals shall be returned to their respective storage cabinets (e.g. flammables in flammable storage cabinet). Works at the Biological Safety Cabinet shall be terminated and all biohazardous materials secured. All doors and windows shall be closed to contain possible fire and smoke.

Upon confirmation of the emergency situation, the duty personnel at the Fire Command Centre will activate the 2nd stage alarm to initiate the evacuation of all personnel within the building. The Fire Alarm will ring continuously on all floors.

Separately, on hearing the Gas Evacuation Alarm (RED Alarm), all staff shall shut down the various machineries and lab equipment, secure the chemicals and biological agents and evacuate from the lab immediately. The Gas Evacuation Alarm is different from and not interlinked with the Fire Alarm. Evacuation is not required during the Stage 1 Verify Gas Leak Alarm (ORANGE Alarm).

During an evacuation, all personnel, including visitors, shall exit the facility via the nearest exit stairway in an orderly manner as guided by the Fire Warden in accordance with the Fire Evacuation Plan (Annex B-1). The lift shall not be used at any time. No personnel shall return to the premise to collect an item or personal belongings during an evacuation. The Fire Warden shall check to ensure that there is no one still remaining within the Scheduled Premise.

All personnel shall then proceed to the Assembly Area (Annex B-2) and wait there for further instruction. The Company Representative shall mark the attendance of all company staff and visitor at the Assembly Area and report attendance to the Building Fire Safety Manager. The SCDF ground commander and Building Fire Safety Manager shall be notified of any unaccounted personnel.

All personnel shall remain at the Assembly Area until instruction to return to the building has been given by the Building Fire Safety Manager or SCDF Ground Commander for fire emergency and the Site Controller for gas emergency.

3.2.2.1  Non-CARES personnel

Non-CARES personnel present on CARES site are either visitors or contractors.  In the event of an evacuation, visitors will be following the instructions of their CARES host.  Contractors are to be handled as described in the CARES contractor procedures.  Briefly, a designated responsible person represents the contractors, who will ensure their complete evacuation and confirm this evacuation to the Site Incident Controller (Lab manager).

### 3.2.3	(Phase II b) Emergency Actions to Mitigate or Contain the emergency

- Emergency Shutdown Procedure

All lab equipment in operation shall be shut down during an emergency prior to an evacuation. Attempt shall be made to either quench or contain chemical reactions that are still in progress. All biohazardous materials shall be secured. All flammable chemicals shall be returned to the flammables storage cabinets and the doors of the cabinets closed.

The gas solenoid valves for the various gas lines should be closed to stop delivery of gases to the respective points of use. However, while attempts have been made to install the gas solenoid valves close to the respective gas cylinders, gas leaks may still occur along the short lengths of pipings and valves between the gas cylinders and the solenoid valves. A more failsafe approach is to turn off the gas cylinder valves. However, this should only be attempted when the designated person has assessed the situation and deemed that this operation would not lead to increased risk or delay the prompt evacuation of all personnel.

- Fire Fighting, Hazmat Monitoring, Containment  and Rescue

The company Site Incident Manager shall lead the emergency response effort for emergencies occurring within the Scheduled Premise only. Trained Company Emergency Response Team (CERT) personnel shall provide support.

In the event that the Site Incident Manager is not available, his responsibilities shall be filled by the CERT.

During a fire emergency, the CERT personnel shall assess the suitability of the firefighting media (dry powder in Type ABC fire extinguishers, CO2 fire extinguishers, and water in hose reel) before attempting to use the media to fight the fire. The fire extinguishers on site are not suitable for Class C fires. One should terminate the flammable gas supply to extinguish the fire.

In the event of a chemical spill, the CERT personnel shall contain and clean up the spill by applying the Spill Containment Protocol as detailed in Annex H.

During a gas leak emergency, the CERT personnel shall ensure that delivery of all gases is promptly terminated. The team shall then take steps to identify the source of the leak and if possible to perform simple repair of the leak. A leaking gas line containing flammable or toxic gases shall be purged with inert gases (e.g. nitrogen (N2) gas) for at least 10 mins prior to the commencement of any repair operation.

The following emergency equipment is located at the Scheduled Premise:

| S/N | Equipment | Capacity | Quantity | Location |
| --- | --- | --- | --- | --- |
| 1. | Type AB fire extinguisher | 4.0 kg | 4 | Lab 1 – High pressure rooms |
|  |  |  | 1 | Lab 1 - Gas Cylinder Room 2 |
|  |  |  | 1 | Lab 1 - Particle Processing Room |
|  |  |  | 1 | Lab 1 - Particle Technology Lab |
|  |  |  | 1 | Gas Cylinder Room 1 |
|  |  |  | 1 | M&E Plant Room |
|  |  |  | 1 | Dry Lab |
|  |  |  | 1 | Lab 2 - Bio-Sustainability Lab |
|  |  | 6.0 kg | 1 | Lab 1 – Chemical Prep Area |
|  |  |  | 2 | Lab 1 - Wet Chemistry Lab |
|  |  |  | 1 | Lab 1 - Control Area |
|  |  |  | 1 | Lab 2 – Advanced Microtechnology Lab |
|  |  |  | 1 | Lab 2 – Electrochemical Analysis Lab |
| 2. | Carbon dioxide (CO2) fire extinguisher | 5.0 kg | 1 | Lab 1 – Chemical Prep Area |
|  |  |  | 2 | Lab 1 - Wet Chemistry Lab |
|  |  |  | 1 | Lab 1 - Control Area |
|  |  |  | 1 | Lab 1 - Particle Processing Room |
|  |  |  | 1 | Lab 1 - Particle Technology Lab |
|  |  |  | 1 | M&E Plant Room |
|  |  |  | 1 | Lab 2 – Advanced Microtechnology Lab |
|  |  |  | 1 | Lab 2 – Electrochemical Analysis Lab |
|  |  |  | 1 | Lab 2 - Bio-Sustainability Lab |
| 3. | Hose reel | - | 1 | Lab 1 – Control Area |
|  |  |  | 1 | Outside facility along corridor |
|  |  |  | 1 | Lab 2 – Electrochemical Analysis Lab |
| 4. | Spill containment kit | - | 1 | Within Lab 1 |
|  |  |  | 1 | Within Lab 2 |
| 5. | Fire hydrant | - | 1 | Outside building along CREATE Way |
|  |  |  | 1 | Outside Enterprise wing on the building (near bus stop) |
| 6. | First Aid Box | - | 1 | Within Lab 1 |
|  |  |  | 1 | Within Lab 2 |
| 7. | Fire Blanket | - | 11 | Within Lab 1 and 2 |

For emergency occurring outside the Scheduled Premise, the company shall only be responsible for performing emergency shutdown of its equipment, secure its chemicals, terminating its gas supplies and evacuating all its personnel from the Scheduled Premise. Firefighting and other emergency response duties shall be solely coordinated by the Building Fire Safety Manager.

3.2.3.3	Procedure to Implement In-Place Protection (IPP)

Note: The signal for IPP will be broadcast over radio or TV after the sounding of important message signal through the Public Warning System. The authorities may also conduct a door-to-door notification.

All personnel shall familiarise themselves with the various signals under the SCDF Public Warning System. In the event that the “Important Message” siren is triggered, personnel shall tune in immediately to any local FM radio station to hear the important broadcast.

Should an IPP be initiated, the Company Site Controller shall assume the role of IPP Coordinator and take lead role in coordinating the IPP effort. All personnel shall stop all activities immediately and gather at the open lab area in Lab 2. All equipment shall first be shut down and valves of all gas cylinders turned off.

To minimise the infiltration of any toxic fumes present in the environment, the air-conditioning and mechanical ventilation system for the premise shall be turned off immediately. Furthermore, the gaps around the windows, perimeter entrance doors as well as the doors leading into the lab shall be sealed using masking tape. All fume hood openings and local fume extractor outlets shall be sealed using plastic wraps and masking tape.

The IPP Coordinator shall establish a communication channel with the Building Fire Safety Manager to update each other of the situation and to implement further actions as required. The radio shall be kept on at all times so that the group may listen for further instructions.

Upon termination of the IPP, the doors shall be opened and the air-conditioning and mechanical ventilation system turned on to ventilate the area.

### 3.2.4	(Phase III) To clean up / decontaminate and resume normal operations.

- Clean up Operations

The Type AB fire extinguishers contain ammonium phosphate and/or ammonium sulfate powders that can be irritating to the eyes, skin and lungs. Caution should thus be observed to avoid breathing in the chemical dust during use as well as during cleanup of the discharge residues. The ABC powders shall be swept up, bagged and placed in a dumpster. The use of water for cleanup should be avoided as the powder becomes pasty and more difficult to pick up when wet. The wet paste can also be mildly corrosive.

The carbon dioxide used in CO2 fire extinguisher is an asphyxiant and may cause asphyxiation when a person is exposed to high level of it. However, carbon dioxide does not leave any residue and no cleanup is required.

For chemical spill, the cleanup shall be performed in accordance with the Emergency Spill Containment Protocol as detailed in Annex H. Contaminated adsorbent materials and Personal Protective Equipment (PPE) shall be bagged and placed within the designated enclosed chemical waste container for collection by licensed toxic industrial waste collector.

Similarly, biological spill cleanup shall be performed in accordance with the Emergency Spill Containment Protocol as detail in the SOP (Annex A). Materials used for biological spill cleanup shall be bagged and disinfected, either using chemical disinfectant or steam sterilisation. This waste shall also be collected by licensed toxic industrial waste collector (pathogenic waste).

For gas leak incidents, the safety committee shall convene an on-site meeting to discuss and execute the most appropriate recovery measure prior to allowing the rest of staff back into the lab.

Our appointed waste collector is Chem-Solv Technologies Pte. Ltd. 29/31 Pioneer Sector 2, Tuas Singapore, 628325 Singapore (Tel: 68614277)

- Other Emergency Plans

No other emergency plan is deemed necessary at this time (Annex E).

## 3.3	GROUPING AND TASKS

The roles and responsibilities of the various appointment holders of this Emergency Response Plan are tabled in Annex F.

## 3.4	KEY PESONNEL EMERGENCY CONTACT NUMBERS

The contact numbers of the key appointment holders are listed in Annex D.

# 4	SERVICE SUPPORT

This section describes the facilities and equipment that are available to assist the Company in mitigating an emergency.

## 4.1	FIXED INSTALLATIONS

### 4.1.1    FIRE / HAZMAT PROTECTION FACILITIES

4.1.1.1	Detection System

Lower Explosive Limit (LEL), carbon monoxide, carbon dioxide, oxygen and hydrogen sulfide gas detectors are installed within the premise to check for gas leaks.

Smoke detectors are installed inside the electrical room to guard against fire.

4.1.1.2	Extinguishment System

The building is sprinkler protected and is equipped with water hose reels located at strategic locations (see Annex B-1 for locations). Multiple fire hydrants are strategically located outside the building. The building is equipped with dry risers at the fire fighting and smoke stop lobbies. No other fixed extinguishment system is present.  Type ABC and CO2 fire extinguishers are located within the Scheduled Premise.

### 4.1.2    SAFETY AND FIRST AID EQUIPMENT

| S/N | Item | Qty | Location |
| --- | --- | --- | --- |
| 1. | First Aid Box (Box A) | 1 | Lab 1 |
|  |  | 1 | Lab 2 |
| 2. | Safety eyewash | 2 | Lab 1 |
|  |  | 1 | Lab 2 |
|  |  | 1 | Inside Advanced Microtechnology Lab (Class 1,000 Cleanroom; Lab 2) |
|  |  | 1 | Bio-Sustainability Lab (Lab 2) |
| 3. | Safety shower | 2 | Lab 1 |
|  |  | 1 | Lab 2 |
| 4. | Chemical spill kit | 1 | Lab 1 |
|  |  | 1 | Lab 2 |
| 5. | Biological spill kit | 1 | Lab 2 |
| 6. | Personal Protective Equipment (e.g. gloves, masks, lab coat) | - | At entrance to both Labs 1 and 2 |

### 4.1.3    OTHER PROTECTION AND GENERAL EQUIPMENT

Chemical fume hoods, canopy hoods and local fume extractors are located within the lab area for handling of volatile hazardous chemicals.

Biological safety cabinets are installed at the Bio-Sustainability Lab for handling of biohazardous materials.

Safety showers and eyewashes are installed to facilitate rinsing during chemical / biological exposure scenarios.

Self Contained Breathing Apparatus is located at the M&E Plant Room for use during investigation of gas leak event.

Potable multi-gas detector is located at the M&E Plant Room for use during investigation of gas leak event.

No other protection equipment is deemed necessary at this time.

# 5	COMMAND AND SIGNAL

This section describes the command and communication structure of the Company during an emergency.

## 5.1	COMMAND STRUCTURE

### 5.1.1    INCIDENT ORGANISATION CHART

During an emergency that occurs within the Scheduled Premise, the Site Incident Controller shall assume lead role in organising the response (firefighting, gas leak event investigation or spill cleanup).

The Site Controller shall be responsible for reporting the incident to the Building Fire Safety Manager. The Building Fire Safety Manager shall be the main party to liaise with SCDF Ground Commander for the building emergency effort. The Site Controller shall provide company specific information and render support when called upon. The specific roles and responsibilities of the various appointment holders are tabled in Annex F. The structure of the Company Emergency Response Team is presented in Annex I.

### 5.1.2    LOCATION AND COMPONENT OF COMMAND CENTRE

The Fire Command Centre is located at the basement level of the CREATE building (at Tower block area).

# 6	PLAN REVIEW & MAINTENANCE

This section details the plan review process. The plan review is to be conducted on an annual basis.

## 6.1	COMMUNICATION OF PLAN

The Site Controller shall be responsible for keeping this Emergency Response Plan current and for communicating the emergency procedures to all staff within the Company. An annual review of the Emergency Response Plan shall be conducted regardless of whether any change has been made.

## 6.2	TABLE TOP EXERCISE

The next table top exercise between Site Controller and Site Incident Controller is scheduled for June, 2018.

## 6.3	CONDUCT OF EMERGENCY DRILL

Key findings from the emergency drills are as follows:

- All evacuation exercises exceeded expectations, with full evacuation of the laboratories occurring in under 20 seconds and assembly at the assembly point in under 2 minutes.

- Spill emergency exercises highlighted the necessity to provide support to casualties during decontamination, and to allow for privacy if a casualty needs to disrobe (to remove contaminated clothing).  For this reason, emergency clothes were located on site and the importance of evacuating and providing a staff member of the same sex to provide support during decontamination were incorporated in the drills.  These lessons were absorbed well by the staff.

## 6.4	REVIEW OF HAZARD RISK ASSESSMENT

Current versions of the respective lab activities risk assessments were reviewed during the preparation of this Emergency Response Plan. All required control measures have been highlighted in this ERP.

Attempts shall be made to review all risk assessment on a periodical basis and key findings from future review shall be recorded here for reference and improvement. Gaps in the current plan shall also be identified and addressed.

# ANNEX A – CREATE BUILDING SITE PLAN

CREATE RESEARCH

WING

Vehicle access to RESEARCH Wing

Evacuation Assembly Point

RESEARCH

WING

Evacuation Assembly Point

# ANNEX B 1 – CREATE BUILDING RESEARCH WING LEVEL 7 PLAN

LEGEND

v

6 nos.

6 nos.

v

90 mins rated Flammable Gas Cabinets (6 nos. at each location)

Safety Eyewash

Safety Shower

Flammables Safety Cabinet

First Aid Kit

Fire Extinguishers (Type ABC)

Emergency Exit from Scheduled Premise

# ANNEX B-2 – LOCATION OF ASSEMBLY AREA

Assembly

Area

NUCLEOS BUILDING

# ANNEX C – CHEMICAL INVENTORY

| LIQUID |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IRP | Chemical | Physical Form | Lab | Cabinet location | Unit Capacity of Packaging / Container (Litres) | Total No. of Packaging / Container | Maximum Qty Stored On-Site (Litres) |
| 1 | 32% HCl | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | Aluminium isopropoxide | Liquid | 1 | FH_01 solvent cabinet | 1 | 1 | 1 |
| 1 | Aluminium isopropoxide 98+% | Liquid | 1 | FH_01 solvent cabinet | 0.5 | 1 | 0.5 |
| 1 | Aluminium sec-butoxide 98% | Liquid | 1 | FH_01 solvent cabinet | 0.5 | 1 | 0.5 |
| 1 | Aluminium tri-sec-butoxide 97% | Liquid | 1 | FH_01 solvent cabinet | 0.5 | 1 | 0.5 |
| 1 | Citric acid | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | Dilute HCl | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | Ethanol | Liquid | 1 | FH_01 solvent cabinet | 1 | 2 | 2 |
| 1 | Magnesium ethoxide | Liquid | 1 | FH_01 solvent cabinet | 1 | 1 | 1 |
| 1 | Methanol | Liquid | 1 | FH_01 solvent cabinet | 1 | 2 | 2 |
| 1 | n-Heptane | Liquid | 1 | FH_01 solvent cabinet | 1 | 1 | 1 |
| 1 | Nitric acid | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | pH 4 buffer solution | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | Propan-2-ol | Liquid | 1 | FH_01 solvent cabinet | 1 | 2 | 2 |
| 1 | Tetraethyl orthosilicate 98% | Liquid | 1 | FH_01 solvent cabinet | 0.5 | 1 | 0.5 |
| 1 | Toluene | Liquid | 1 | FH_01 solvent cabinet | 1 | 1 | 1 |
| 1 | Universal indicator solution | Liquid | 1 | FH_02 acid cabinet | 1 | 1 | 1 |
| 1 | Various NaOH and HNO3 stock solutions | Liquid | 1 | FH_01 solvent cabinet | 1 | 2 | 2 |
| 1 | Zirconium isobutoxide, 80% solution in isobutanol | Liquid | 1 | FH_01 solvent cabinet | 0.5 | 1 | 0.5 |
| 2 | (3-Aminopropyl)trimethoxysilane ＊ 3 | Liquid | 2 | Electrochem | 0.1 | 3 | 0.3 |
| 2 | (3-Glycidoxypropyl)trimethoxysilane | Liquid | 2 | Electrochem | 1 | 20 | 20 |
| 2 | 1,1,2-trichloroethane | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | 1,2-Dibromoethane | Liquid | 2 | Electrochem | 0.01 | 2 | 0.02 |
| 2 | 1,2-Dichloroethane | Liquid | 2 | Electrochem | 2.5 | 4 | 10 |
| 2 | 1-Butanethiol | Liquid | 2 | Electrochem | 0.25 | 1 | 0.25 |
| 2 | 1-Butanol | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | 1-Dodecanethiol | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | 1-Dodecanethiol | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | 1-Dodecanol | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | 1-Methoxy-2-propanol acetate | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | 1-Methyl-2-pyrrolidinone | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | 1-Octanethiol | Liquid | 2 | Electrochem | 0.25 | 1 | 0.25 |
| 2 | 1-Octanol | Liquid | 2 | Electrochem | 2.5 | 1 | 2.5 |
| 2 | 1-Thioglycerol | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | 2,2,4-Trimethylpentane | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | 2-Methyl-2-propanol | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | 3-(Trimethoxsilyl)propyl methacrylate | Liquid | 2 | Electrochem | 0.1 | 4 | 0.4 |
| 2 | 3-Aminopropyltriethoxysilane | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | Acetone | Liquid | 2 | Electrochem | 4 | 12 | 50 |
| 2 | Acetonitrile | Liquid | 2 | Electrochem | 4 | 12 | 50 |
| 2 | Allyltrimethoxysilane | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Ammonia solution | Liquid | 2 | Electrochem | 2.5 | 1 | 2.5 |
| 2 | Ammonium hydroxide | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Benzyl chloride | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | Bisphenol A glycerolate (1 glycerol/phenol) diacrylate | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Brandford reagent | Liquid | 2 | Biological Containment Area | 0.5 | 1 | 0.5 |
| 2 | Butan-1-ol | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Butylferrocene | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | Chloroform | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Chlorotrimethylsilane | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | Chlorotrimethylsilane | Liquid | 2 | Electrochem | 0.25 | 1 | 0.25 |
| 2 | Crystal violet | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Cyclohexane | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Dichloroacetic acid | Liquid | 2 | Electrochem | 0.01 | 1 | 0.001 |
| 2 | Dichloromethane | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Efhard | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Ethylene glycol | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Formic acid | Liquid | 2 | Biological Containment Area | 1 | 1 | 1 |
| 2 | Glycerol | Liquid | 2 | Biological Containment Area | 1 | 5 | 5 |
| 2 | Gold chloride solution 200 mg/dL | Liquid | 2 | Biological Containment Area | 0.5 | 1 | 0.5 |
| 2 | HEADSTOCK LUBRICANT | Liquid | 2 | Electrochem | 1 | 4 | 4 |
| 2 | Hexane | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | HY 219 (an epoxy glue) | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Hydrochloric acid 37% | Liquid | 2 | Electrochemical Analysis Area/Biological Containment Area/Class 1000 Clean Room | 1 | 10 | 10 |
| 2 | Isopropanol | Liquid | 2 | Electrochem | 4 | 12 | 50 |
| 2 | LUDOX® HS-40 colloidal silica | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | LUDOX® HS-40 colloidal silica | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Methanol | Liquid | 2 | Class 1000 Clean Room | 4 | 12 | 50 |
| 2 | Mineral oil | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Mineral oil | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Minimum essential medium eagle | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | N, N, N', N'-tetramethylethylene diamine | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | N,N-dimethylformamide | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Nitric acid | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Nitrobenzene | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Octadecylmercaptan | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | Olive oil | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | o-Tolidine solution | Liquid | 2 | Biological Containment Area | 0.5 | 1 | 0.5 |
| 2 | Paraffin oil | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Phenolphthalein | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | Phosphoric acid | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Poly(ethylene glycol) | Liquid | 2 | Electrochem | 0.25 | 1 | 0.25 |
| 2 | Polytetrafluoroethylene | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | Propan-2-ol | Liquid | 2 | Electrochem | 2.5 | 3 | 7.5 |
| 2 | Proplyene carbonate, 99.5% | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Pyrrole | Liquid | 2 | Electrochem | 0.05 | 1 | 0.05 |
| 2 | Siliocne elastomer curing agent | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Sodium silicate solution | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Sodium sulfate standard solution | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Span 80 | Liquid | 2 | Electrochem | 0.25 | 1 | 0.25 |
| 2 | SU-8 2000 thinner | Liquid | 2 | Class 1000 Clean Room | 1 | 5 | 5 |
| 2 | SU-8 2002 | Liquid | 2 | Class 1000 Clean Room | 1 | 5 | 5 |
| 2 | SU-8 2025 | Liquid | 2 | Class 1000 Clean Room | 1 | 5 | 5 |
| 2 | SU-8 2100 | Liquid | 2 | Class 1000 Clean Room | 1 | 5 | 5 |
| 2 | SU-8 3025 | Liquid | 2 | Class 1000 Clean Room | 1 | 5 | 5 |
| 2 | Sulfuric acid | Liquid | 2 | Electrochemical Analysis Area/Biological Containment Area/Class 1000 Clean Room | 1 | 10 | 10 |
| 2 | TBAH | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | tert-Butanol | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Tetra(ethylene glycol) diactrylate | Liquid | 2 | Electrochem | 0.02 | 1 | 0.02 |
| 2 | Tetraethyl orthosilicate | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Tetraethyl orthosilicate | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Tetrahydrofuran | Liquid | 2 | Electrochem | 1 | 10 | 10 |
| 2 | Thymine | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | Toluene | Liquid | 2 | Electrochem | 1 | 5 | 5 |
| 2 | Tricine | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 2 | Tris-methylamine | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Zetasperse 2100 | Liquid | 2 | Electrochem | 0.5 | 1 | 0.5 |
| 2 | Zinc sulfate heptahydrate | Liquid | 2 | Electrochem | 1 | 1 | 1 |
| 2 | α,α,α-Trifluorotoluene | Liquid | 2 | Electrochem | 0.1 | 1 | 0.1 |
| 3 | (Methylcyclopentadienyl)manganese(I) tricarbonyl | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | 1,2,3,4-Tetrahydronaphthalene 99% | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | 1,2,4-Trimethylbenzene,98% | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | 1-Butanol | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | 1-Methylnaphthalene,ampule of 100 mg | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | 1-Phenyldecane,puriss. p.a., standard for GC, >=99.5% (GC) | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | 2,2,4,4,6,8,8-Heptamethylnonane,98% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | 2,2,4-Trimethylpentane,anhydrous, 99.8% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Acetic acid | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Acetone | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 4 | 4 |
| 3 | Acetylacetone | Liquid | 1 | FH_03/04 solvent cabinet | 2.5 | 1 | 2.5 |
| 3 | Aluminum tri-sec butoxide | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Biodiesel | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Butylbenzene,>=99% | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Cyclohexane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Decahydronaphthalene, mixture of cis + trans98% | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Diacetone alcohol | Liquid | 1 | FH_03/04 solvent cabinet | 2.5 | 1 | 2.5 |
| 3 | Diesel | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Diethyl ether | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Ethanol | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Ethyl acetate | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Gasoline | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Germanium(IV) chloride | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Heptane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Hexadecane | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Hexadecane, 100 ml | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Hexamethyldisilazane(HMDS) | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Hexane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Iron(0) pentacarbonyl | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Isopentane | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Isopropanol | Liquid | 1 | FH_03/04 solvent cabinet | 2 | 2 | 4 |
| 3 | Lauric acid,natural, >=98%, FCC, Kosher | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Methanol | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Methyl ethyl ketone (MEK) | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Methyl glycol ether | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Methyl isobutyl ketone (MIBK) | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Methylcyclohexane >=99.5% (GC) | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Myristic acid,Sigma Grade, >=99% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | nPentane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | n-propyl alcohol | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Palmitic acid,SigmaUltra, >=99% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Phenylcyclohexane,>=97% | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 2 | 1 |
| 3 | Phosphorus trichloride | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Polyethylene glycol | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Stearic acid,reagent grade, 95% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Tetradecane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Tetraethyl orthosilicate (TEOS) | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Tin(IV) chloride | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Titanium (IV) butoxide | Liquid | 1 | FH_03/04 solvent cabinet | 0.5 | 1 | 0.5 |
| 3 | Titanium tetrachloride (TiCl4) | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Titanium tetraisopropoxide (TTIP) | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Toluene,anhydrous, 99.8% | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 2 | 2 |
| 3 | Undecane | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | VANADIUM(V) OXYTRICHLORIDE | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 1 | 1 |
| 3 | Waste solvent waiting for disposal | Liquid | 1 | FH_03/04 solvent cabinet | 1 | 4 | 4 |

| SOLID |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IRP | Chemical | Physical Form | Lab | Cabinet location | Unit Capacity of Packaging / Container (Litres) | Total No. of Packaging / Container | Maximum Qty Stored On-Site (kg) |
| 1 | Aluminium acetate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Aluminium chloride hexahydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Aluminium nitrate nonahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Aluminium oxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium carbonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium iron (III) sulphate dodecahydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium metavanadate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium molybdate tetrahydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Ammonium niobate (V) oxalate hydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Bentonite powder | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Benzenetricarboxylic acid | Solid | 1 | Oxidising Agent Storage | 1 | 1 | 1 |
| 1 | Calcium bentonite | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium carbonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium hydroxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium nitrate tetrahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Calcium oxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium sulphate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium sulphate dihydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Calcium titanate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Carbon black | Solid | 1 | Combustion Lab Chemical storage | 0.1 | 2 | 0.2 |
| 1 | Cerium chloride heptahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Cerium nitrate hexahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Chromium nitrate nonahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Citric acid monohydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Cobalt nitrate hexahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Copper (II) acetate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Copper (II) acetylacetonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Copper chloride hihydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Copper nitrate hemipentahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Copper oxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Croconic acid | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Cupric sulphate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Electrode storate solution | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Epoxy resin kit | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Gadolinium (III) chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Gallium nitrate hydrate | Solid | 1 | Oxidising Agent Storage | 1 | 1 | 1 |
| 1 | Graphite powder | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Hexadecanediol | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Hydrogen peroxide | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Iron (II) chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron (III) chloride anhydrous | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron (III) chloride hexahydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron (III) oxide powder | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron (III) sulphate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron acetate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Iron acetylacetonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Iron nitrate nonahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Iron oxide (magnetite) | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron powder | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Iron wire | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Iron-chrome based high temperature gas shift catalyst | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Kaolin | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Magnesium acetate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Magnesium carbonate hydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Magnesium formate dihydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Magnesium nitrate hexahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Magnesium oxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Manganese nitrate hydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Nickel nitrate hexahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Oleylamine | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Oxalic acid | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Palladium acetate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Phthalic acid | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Polyethylene oxide | Solid | 1 | Oxidising Agent Storage | 1 | 1 | 1 |
| 1 | Potassium bromide | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Potassium carbonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Potassium chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Potassium hydroxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Potassium nitrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Rubidium chloride | Solid | 1 | Combustion Lab Chemical storage | 0.5 | 1 | 0.5 |
| 1 | Ruthenium chloride hydrate | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Silica (chromatography grade) | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Silica (precipitated) | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Silica nanoparticles | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Silica-alumina catalyst | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Silver chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Soda lime | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Sodium bicarbonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Sodium carbonate | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Sodium chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Sodium hydroxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Sodium nitrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Sodium sulphate (anhydrous) | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Titania nanoparticles | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Titanium (IV) chloride | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Titanium (IV) ethoxide | Solid | 1 | Combustion Lab Chemical storage | 1 | 1 | 1 |
| 1 | Urea | Solid | 1 | Combustion Lab Chemical storage | 1 | 2 | 2 |
| 1 | Yttrium nitrate hexahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 1 | 1 |
| 1 | Zirconium nitrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Zirconyl (IV) nitrate | Solid | 1 | Oxidising Agent Storage | 1 | 2 | 2 |
| 1 | Zirconyl chloride octahydrate | Solid | 1 | Oxidising Agent Storage | 1 | 1 | 1 |
| 2 | (-)-Riboflavin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | (-)-Riboflavin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | 1,10-Phenanthrolin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | 11-Mecraptoundecanoic Acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 1-Hydroxycyclohexyl phenyl ketone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | 1-Naphthyl phosphate calcium salt trihydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | 2,2′-Azino-bis(3-ethylbenzothiazoline-6-sulfonic acid) diammonium salt | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | 2,2-Bipyrimidine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | 2,2-Dimethoxy-2-phenylacetophenone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | 2,2'-Dipyridyl | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 2,4-Dinitrofluorbenzol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 2,4-Dipyridyl | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 2,5-Dichloro-1,4-benzoquinone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0002 |
| 2 | 2,6-Dichlorophenolindophenol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 2-Benzyl-2-2(dimethylamino)-4'-morpholinobutyrophenone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | 2-Naphthol (β) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 3-(N-morpholino propanesulfonic acid) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | 3,3'-(Ethylenedioxyl)diphenol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | 3,6-Di-2-pyridyl-1,2,4,5-tetrazine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | 3-Aminophthalhydrazide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 4,4'-Bipyridyl | Solid | 2 | Electrochemical Analysis Area |  |  | 0.003 |
| 2 | 4,4'-Dipyridyl | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 4-Cyanopyridine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 4-Iodophenol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | 6-Aminofluorescein | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | 8-Anilino-1-naphthalenesulfonic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | Acrylamide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Activated carbon | Solid | 2 | Electrochemical Analysis Area |  |  | 0.3 |
| 2 | Adenosine 5'-triphosphate disodium salt hydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Agar | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Agarose | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Aldrithiol 4 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.003 |
| 2 | Aluminium oxide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Aluminium potassium sulphate dodecahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ammonium bicarbonate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ammonium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Ammonium ferric citrate green | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ammonium hydrogenphosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Ammonium molybdate tetrahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Ammonium oxalate monohydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 2 |
| 2 | Ammonium persulfate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Ammonium sulphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Ammonium tetrachloropalladate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Aniline | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Aniline | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Anthracen | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Anthracene-9-carbonyl chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Arthrospira maxima | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Barium carbonate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | bathophenanthroline | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Biotin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0001 |
| 2 | Biotin | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Bis-(diphenylphosphino)methane | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Bispentamethylcyclopentadienyiron | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0005 |
| 2 | Boric acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Bromine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Cadmium acetate dihydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.08 |
| 2 | Caffeine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Calcium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Calcium chloride dihydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Calcium hydride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Calcium ionophore A23187 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.000005 |
| 2 | Calcium nitrate tetrahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Calcium pantothenate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Carbon Black | Solid | 2 | Electrochemical Analysis Area |  |  | 0 |
| 2 | Carbon Black | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Carbon Graphite | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Carbon NanoTubes | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Carmine | Solid | 2 | Electrochemical Analysis Area |  |  | 0 |
| 2 | Chlorella vulgaris | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Chrome | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Chroogloeocystis siderophila | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Citric acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Cobalt (II) chloride hexahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Cobalt (II) nitrate hexahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Cobalt chloride hexahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Cobaltocenium hexafluorophosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Copper (II) nitratehemipentahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.015 |
| 2 | Copper (II) Sulfate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Copper (II) sulfate pentahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Copper sulfate pentahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Copstone | Solid | 2 | Electrochemical Analysis Area |  |  | 0 |
| 2 | Cupferron | Solid | 2 | Electrochemical Analysis Area |  |  | 0.015 |
| 2 | Cupriavidus metallidurans | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | CY 219 | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | cyanocobalamin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Cyanocobalamin | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Cytochrome C | Solid | 2 | Electrochemical Analysis Area |  |  | 0.00001 |
| 2 | D-(+)-Glucose | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | DILUTED ETHANOL*2 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Dioctylsulfosuccinate calcium salt | Solid | 2 | Electrochemical Analysis Area |  |  | 0.03 |
| 2 | Dioctylsulfosuccinate sodium salt | Solid | 2 | Electrochemical Analysis Area |  |  | 0.03 |
| 2 | Disodium Phosphate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Disodium tetraborate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Diuron | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | DL-dithiothreitol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | DL-Malic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | DL-Malic acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Dodecylamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | D-pantothenic acid hemicalcium | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | DY 219 | Solid | 2 | Electrochemical Analysis Area |  |  | 3 |
| 2 | Erythromycin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.015 |
| 2 | Ethylene glycol-bis(2-amino-ethylether)-N,N,N',N'-tetraacetic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Ethylenediaminetetraacetic acid disodium salt dihydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Europium (III) Chloride hexahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Ferric chloride hexahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ferric citrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ferric citrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Ferrocene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Ferrocenecarboxylic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0002 |
| 2 | Ferrocenemethanol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0005 |
| 2 | Ferrocenium hexafluorophosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | ferrous sulphate heptahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Fluorescein | Solid | 2 | Electrochemical Analysis Area |  |  | 0.03 |
| 2 | Folic acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Gold | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Gold (III) Chloride trihydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Graphene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Hexaamineruthenium (II) chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Hexaamineruthenium (III) chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | hexaflurophosphoric ferric citrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Hydrazine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Hydrogen hexachloroplatinate (IV) hydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Hydroquinone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Hydroquinone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Hydroxyl naphthol blue | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | hydroxylamine hydrichloride | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Inositol | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Iron (II) sulfate heptahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Iron (III) Chloride hexahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Iron (III) ionophore IV | Solid | 2 | Electrochemical Analysis Area |  |  | 0.00005 |
| 2 | L-ascorbic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | L-Cysteine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Lead (II) carbonate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Lithium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Lithium fluoride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | Magnesium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Magnesium sulfate heptahydrate | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Manganese (II) chloride tetrahydrate | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Manganese (II) sulfate monohydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Manganese (II) Sulfate Morohydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Manganese sulfate monohydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Meat extract | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Meat extract | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | MES hydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Methyl viologen dichloride hydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Methylene blue hydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Methyltrioxorhenium(VII） | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0001 |
| 2 | Metolat 392 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Molecular sieve dehydrate fluka with indicator for drying slovents | Solid | 2 | Electrochemical Analysis Area |  |  | 0.15 |
| 2 | Molecular sieve UOP type 3A | Solid | 2 | Electrochemical Analysis Area |  |  | 0.2 |
| 2 | Molecular sieves 4 A | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Molybdenum haxacarbonyl | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Monopotassium Phosphate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | MOPS | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Myo-inositol | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | N-(3-dimethylaminopropyl)-N'-ethyl-carbodiimide hydrochloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | N,N,N',N'-tetramethyl-p-phenylenediamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | N,N,N',N'-tetramethyl-p-phenylenediamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | N,N'-bis(3-methylphenyl)-N,N'-diphenylbenzidine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | N,N'-bis(salicylidene)-ethylenediamino-colbat (II) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | N,N'-dicyclohexylcarbodiimide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | N,N'-dimethyl-9,9'-biacridinium dinitrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | N,N'-methylenebisacrylamide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | N-hydroxysuccinimide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Nicotinamide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Nicotinamide | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Nicotine acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Nicotinic acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Nikel (II) sulfate hexahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Nile blue | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Nitroacetic Acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Nitroacetic Acid | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Palladium (II) chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Penicillin G | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Pepstatin A | Solid | 2 | Electrochemical Analysis Area |  |  | 0.000025 |
| 2 | Peptone | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Peptone | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Periodic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Peroxidase | Solid | 2 | Electrochemical Analysis Area |  |  | 0.000005 |
| 2 | Perylene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | Phenanthrene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Phenol | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Phosphate buffered saline tablets | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Phosphorus pentoxide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Platinum | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Poly(9-vinylcarbazole) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Poly(Vinyl butyral-co-vinyl alcohol-co-vinyl acetate) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Potassium bromide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Potassium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Potassium dihydrogen phosphate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Potassium dihydrogenphosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.2 |
| 2 | Potassium Ferricyanide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Potassium Hexacyanoferrate(III) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Potassium Hexacyanoferrate(III) | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Potassium hydroxide | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Potassium iodide bioxtra | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Potassium nitrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Potassium Permanganate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Potassium phosphate dibasic | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Potassium phosphate dibasic anhydrous | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Potassium phosphate dibasic trihydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | p-Toluene sulfonic acid monohydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Pyrazine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | R. Palustris | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Rhodamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Rhodamine 110 chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.00025 |
| 2 | Rhodamine B | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Riboflavin | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Rodamine 123 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.000005 |
| 2 | Rubrene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.0001 |
| 2 | Sea salt | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | sea salt (synthetic) | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Sea Salts | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Shewanella oneidensis | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Silica particles | Solid | 2 | Electrochemical Analysis Area |  |  | 0.2 |
| 2 | Silicon Dioxide | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Silver | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Silver nitrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Silver nitrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Silver powder | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Sodium bicarbonate | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Sodium bis(2-ethylhexyl) sulfosuccinate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Sodium borate decahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Sodium carbonate anhydrous | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Sodium chloride | Solid | 2 | Biological Containment Area |  |  | 10 |
| 2 | Sodium dihydrogen phosphate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Sodium dodecyl benzenesulfonate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Sodium dodecyl sulfate | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Sodium fluoride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Sodium hexafluorophosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Sodium Hydrogen Phosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Sodium hydroxide | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Sodium hypophosphite monohydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.25 |
| 2 | Sodium L-lactate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | Sodium L-lactate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Sodium metaborate tetrahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Sodium molybdate dihydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Sodium nitrate | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Sodium orthovanadate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Sodium perchlorate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Sodium phosphate dibasic | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Sodium phosphate dibasic dihydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 1 |
| 2 | Sodium phosphate dibasic dihydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Sodium pyruvate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.015 |
| 2 | Sodium thiosulphate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Spherical silica | Solid | 2 | Electrochemical Analysis Area |  |  | 0.002 |
| 2 | Stearic acid calcium salt | Solid | 2 | Electrochemical Analysis Area |  |  | 0.2 |
| 2 | Sudan black B | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Sudan III | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Surfynol DF 58 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Synechococcus elongatus | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Synechococcus sp. | Solid | 2 | Biological Containment Area |  |  | 0.2 |
| 2 | Tannic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | TES | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Tetrabutylammonium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetrabutylammonium hexafluorophosphate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetrabutylammonium perchlorate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetrabutylammonium tetraphenylborate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetrachlor-p-benzochinon | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Tetracycline | Solid | 2 | Electrochemical Analysis Area |  |  | 0.025 |
| 2 | Tetraethylammonium tetrafluoroborate 99% | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetrakis (hydroxymethyl)phosphonium chloride solution | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Tetramethylammonunium chloride | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Thiamine hydrochloride | Solid | 2 | Biological Containment Area |  |  | 0.25 |
| 2 | Thianthrene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.02 |
| 2 | Thymine | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Titanium | Solid | 2 | Class 1000 Clean Room |  |  | 0.5 |
| 2 | Trans-1,2-di-(4-pyridyl)-ethylene | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Trans-1,2-dibromocyclohexane | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Trichloroacetic acid | Solid | 2 | Electrochemical Analysis Area |  |  | 0.05 |
| 2 | Tricine | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Triethylamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.001 |
| 2 | Tris(2,2'-bipyridyl)ruthenium(II) chloride hexahydrate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.01 |
| 2 | Tris-4-bromophenylamine | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Tris-4-bromophenylamminium hexachloroantimonate | Solid | 2 | Electrochemical Analysis Area |  |  | 0.005 |
| 2 | Tryptone soya broth | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Tryptone soya yeast extract broth | Solid | 2 | Electrochemical Analysis Area |  |  | 0.5 |
| 2 | Vitamin B12 | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 2 | Yeast extract | Solid | 2 | Biological Containment Area |  |  | 0.25 |
| 2 | Zetasperse 2300 | Solid | 2 | Electrochemical Analysis Area |  |  | 0.1 |
| 2 | Zinc Chloride | Solid | 2 | Biological Containment Area |  |  | 1 |
| 2 | Zinc sulfate heptahydrate | Solid | 2 | Biological Containment Area |  |  | 0.5 |
| 3 | Ammonium nitrate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Cadmium chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.1 |
| 3 | Cadmium nitrate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Cellulose acetate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Cobalt(II) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Ethyl cellulose ether | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Glass ballotini (< 70 micron) | Solid | 1 | Powder Processing Room |  |  | 2 |
| 3 | Glass ballotini (≈200 micron) | Solid | 1 | Powder Processing Room |  |  | 2 |
| 3 | Glass ballotini (≈500 micron) | Solid | 1 | Powder Processing Room |  |  | 1 |
| 3 | Granulac 140 (lactose) | Solid | 1 | Powder Processing Room |  |  | 50 |
| 3 | Granulac 230 (lactose) | Solid | 1 | Powder Processing Room |  |  | 50 |
| 3 | Granulac 70 (lactose) | Solid | 1 | Powder Processing Room |  |  | 50 |
| 3 | Maltodextrin | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Methyl orange | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.1 |
| 3 | Naphthalene,scintillation grade, >=99% | Solid | 1 | Powder Processing Room |  |  | 0.25 |
| 3 | Nickel(II) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Niobium (V) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.05 |
| 3 | Poly (vinyl alcohol) | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Polystyrene | Solid | 1 | Combustion Lab Chemical storage |  |  | 1 |
| 3 | Potassium perchlorate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Silica nanoparticles | Solid | 1 | Powder Processing Room |  |  | 1 |
| 3 | Sulphur | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Titania nanoparticles | Solid | 1 | Powder Processing Room |  |  | 1 |
| 3 | Titanium (IV) oxysulfate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Titanium (IV) sulfate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Tungsten (VI) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.01 |
| 3 | Yttrium (III) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.05 |
| 3 | Zinc nitrate hexahydrate | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |
| 3 | Zirconium(IV) chloride | Solid | 1 | Combustion Lab Chemical storage |  |  | 0.5 |

| GAS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Chemical | Properties | Physical Form | Gas Cylinder Room | Unit Capacity of Packaging / Container (Litres) | Total No. of Packaging / Container | Maximum Qty Stored On-Site (Litres) |
| LP Cylinder Air | Air (22 atm) | Inert | Gas | 1 | 50 | 1 | 50 |
| LP Ar | Ar | Inert | Gas | 1 | 50 | 1 | 50 |
| LP He | He | Inert | Gas | 1 | 50 | 1 | 50 |
| LP N2 | N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP O2 | O2 | Oxidiser | Gas | 1 | 50 | 1 | 50 |
| LP H2 | H2 | Flammable | Gas | 1 | 50 | 1 | 50 |
| LP CH4 | CH4 | Flammable | Gas | 1 | 50 | 1 | 50 |
| LP C2H2 | C2H2 == acetylene | Flammable | Gas | 1 | 50 | 1 | 50 |
| LP C2H4 | C2H4 == ethylene == ethene | Flammable | Gas | 1 | 50 | 1 | 50 |
| LP C2H6 | C2H6 == ethane | Flammable | Gas | 1 | 50 | 1 | 50 |
| LP C3H8 | C3H8 == propane | Flammable (liquified) | Gas | 1 | 9 | 1 | 9 |
| LP CO | CO | Toxic & Flammable | Gas | 1 | 20 | 1 | 20 |
| LP CO2 | CO2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP H2S | 100ppm H2S in N2 | Toxic & Corrosive | Gas | 1 | 10 | 1 | 10 |
| LP O2 Special (5% in He) | 5% O2 in He | Inert | Gas | 1 | 50 | 1 | 50 |
| LP H2 Special (4.5% in He) | 4.5% H2 in He | Inert | Gas | 1 | 50 | 1 | 50 |
| LP O2 Special (5% in N2) | 5% O2 in N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP H2 Special (5% in N2) | 5% H2 in N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP CH4 Special (5% in N2) | 5% CH4 in N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP C2H6 Special (4.4% in N2) | 4.4% C2H6 in N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| LP CO Special (10% in N2) | 10% CO in N2 | Toxic | Gas | 1 | 20 | 1 | 20 |
| LP CO2 Special (20% in N2) | 20% CO2 in N2 | Inert | Gas | 1 | 50 | 1 | 50 |
| HP1 CO | CO | Toxic & Flammable | Gas | 2 | 20 | 1 | 20 |
| HP1 Inert 1 | One of: Ar (Inert) N2 (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP1 CO2 | CO2 | Inert | Gas | 2 | 50 | 1 | 50 |
| HP1 Inert 2 | One of: N2 (Inert) He (Inert) Ar (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP1 H2 | H2 | Flammable | Gas | 2 | 50 | 1 | 50 |
| HP1 Flammable 1 | One of: CH4 (Flammable) C2H4 (Flammable) C2H6 (Flammable) | Flammable | Gas | 2 | 50 | 1 | 50 |
| HP2 CO Special 1 | One of: CO (Toxic & Flammable) 10% CO in N2 (Toxic) | Toxic & Flammable | Gas | 2 | 20 | 1 | 20 |
| HP2 O2 | O2 | Oxidiser | Gas | 2 | 50 | 1 | 50 |
| HP2 Cylinder Air | One of 5% O2 in N2 (Inert) Air (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP2 CO2 | CO2 | Inert | Gas | 2 | 50 | 1 | 50 |
| HP2 Inert 1 | One of: Ar (Inert) N2 (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP2 H2 Special 1 | One of H2 (Flammable) 5% H2 in N2 (Inert) | Flammable or Inert | Gas | 2 | 50 | 1 | 50 |
| HP2 Flammable 1 | One of: CH4 (Flammable) C2H4 (Flammable) C2H6 (Flammable) 5% CH4 in N2 (Inert) 5% C2H4 in N2 (Inert) 5% C2H6 in N2 (Inert) | Flammable or Inert | Gas | 2 | 50 | 1 | 50 |
| HP3 CO | CO | Toxic & Flammable | Gas | 2 | 20 | 1 | 20 |
| HP3 Inert 1 | One of: Ar (Inert) N2 (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP3 CO2 | CO2 | Inert | Gas | 2 | 50 | 1 | 50 |
| HP3 Inert | One of: N2 (Inert) He (Inert) Ar (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| HP3 H2 | H2 | Flammable | Gas | 2 | 50 | 1 | 50 |
| HP3 Flammable 1 | One of: CH4 (Flammable) C2H4 (Flammable) C2H6 (Flammable) | Flammable | Gas | 2 | 50 | 1 | 50 |
| LP (PP) CO Special 1 | One of: CO (Toxic & Flammable) 10% CO in N2 (Toxic) | Toxic & Flammable | Gas | 2 | 20 | 1 | 20 |
| LP (PP) O2 | O2 | Oxidiser | Gas | 2 | 50 | 1 | 50 |
| LP (PP) Cylinder Air | One of: Air (Inert) 5% O2 in N2 (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| LP (PP) CO2 | CO2 | Inert | Gas | 2 | 50 | 1 | 50 |
| LP (PP) Inert 1 | One of: Ar (Inert) N2 (Inert) | Inert | Gas | 2 | 50 | 1 | 50 |
| LP (PP) H2 Special 1 | One of H2 (Flammable) 5% H2 in N2 (Inert) | Flammable or Inert | Gas | 2 | 50 | 1 | 50 |
| LP (PP) Flammable 1 | One of: CH4 (Flammable) C2H4 (Flammable) C2H6 (Flammable) 5% CH4 in N2 (Inert) 5% C2H4 in N2 (Inert) 5% C2H6 in N2 (Inert) | Flammable or Inert | Gas | 2 | 50 | 1 | 50 |

# ANNEX D - PREMISES EMERGENCY DATA

Name of Premise: 	Cambridge Centre for Advanced Research and Education in Singapore Ltd.

Address of Premise: 	1 CREATE Way #07-06/07/08 Research Wing Singapore 138602

Date of Last Survey: 15/02/2016    Conducted By: Derrick Tan

1.  	THREAT ASSESSMENT

HRI Class: HFLP   ISP     FMP   BAP    RAP    Non-HRI

| Risks present: | Sensitive Installations | Flammable Materials | TICs |
| --- | --- | --- | --- |
|  | Biological Agents | Radiological Agents | (Tick where applicable) |

Abbreviations:

- HFLP = High Fire Load Premises

- ISP = Important & Sensitive Premises

- TICP = Toxic Industrial Chemical Premises

- FMP = Flammable Material Premises

- BAP = Biological Agent Premises

- RAP = Radiological Agent Premises

- TIC = Toxic Industrial Chemical

2.  	Occupancy LOAD

| Location (eg. Blk 1, Main Building etc) | Peak Hrs (eg. 0800hrs-1700hrs) | No. of Occupants (Peak Hrs) | Non-Peak Hrs (eg. 1700hrs-0800hrs) | No. of Occupants (Non- Peak Hrs) |
| --- | --- | --- | --- | --- |
| Laboratory | 0900 – 1800 | 25 | 1801 – 0859 | 0* |
| Total Premises Occupancy | 0900 – 1800 | 25 | 1801 – 0859 | 0* |

* Researchers may work overtime periodically.

3.  	OCCUPANCY USAGE & KEY ACTIVITIES

| Block / Tower | Level | Usage & Key Activities |
| --- | --- | --- |
| Research Wing | 7 (Unit #07-06/07/08) | Chemical engineering research |

4.  	Key Personnel Contact Directory

| Name | Designation | Emergency Appointment | Tel No. (Day) | Tel No. (Night) | H/P No. |
| --- | --- | --- | --- | --- | --- |
| - | Security Office | - | 6601 2633 | - | - |
| - | NUS Fire Command Centre | - | 6601 2630 6684 0616 | - | - |
| Markus Kraft | Director | Company Site Controller | 6601 5441 | - | 9633 6709 |
| Hugo Schmidt | Lab Manager | Site Incident Controller |  | - | 9102 0484 |

5. 	IN-HOUSE ERT RESOURCES

| S/No | Manpower | Capabilities |
| --- | --- | --- |
| 1. | Hugo Gerald Schmidt | Trained in gas delivery system safe handling (at U of Cambridge) and attended Occupational First Aid with CPR+AED course |
| 2. | Sheng Yuan | Attended Occupational First Aid with CPR+AED course |

| S/No | Equipment | Quantity |
| --- | --- | --- |
| 1. | Fire extinguishers (Type ABC; 4.0 kg) | 11 |
| 2. | Fire extinguishers (Type ABC; 6.0 kg) | 6 |
| 3. | Fire extinguishers (CO2; 5.0 kg) | 10 |
| 4. | First Aid Kit (Box A) | 2 |
| 5. | Chemical Spill Kit | 2 |
| 6. | Biological Spill Kit | 1 |
| 7. | Safety Eyewash cum Shower | 3 |
| 8. | Safety Eyewash | 2 |
| 9. | Self contained breathing apparatus | 2 |
| 10. | Portable multi-gas detector | 1 |

| S/No | Appliances | Capabilities |
| --- | --- | --- |
| 1. | None |  |

6.  	FIRE RISKS

| Location | Description | Contents & Quantity | UN No. & Hazchem Code | Key Protection Systems |
| --- | --- | --- | --- | --- |
| Lab | Flammable chemicals | Please refer to chemical list in Annex C | Please refer to Annex C | Stored within flammables storage cabinets |
| Gas Cylinder Rooms 1 & 2 | Flammable gases | Please refer to gas list in Annex C | Please refer to Annex C | Stored within 90 min fire rated gas cabinets |

7.   	FIRE PROTECTION SYSTEMS

#### a    Fire Command Centre

| Location | PA/Intercom System | Main Alarm Panel in FCC |
| --- | --- | --- |
| Basement | Yes | Yes |

#### b    Fire Alarm System

| S/No. | Locations of Main Alarm Panel | Locations of Sub-Alarm Panels |
| --- | --- | --- |
| 1. | Within Fire Command Centre | At Fireman Lift Lobby |

#### c    Emergency RESPONSE Plan (ERP)

| S/No. | Location of ERP | Assembly Area | No. of Fire Wardens |
| --- | --- | --- | --- |
| 1. | Within office area | Town Green | 2 |
| 2. | Building Fire Emergency Plan at FCC | Town Green | N/A |

#### d	Fire Suppression Systems

| S/No | Detector Type | Availability | Location |
| --- | --- | --- | --- |
| 1 | Sprinkler | Yes / No | Throughout building |
| 2 | CO / Halon / Fm200 System | Yes / No | NA |
| 3 | Sprinkler Pump Room | Yes / No (Centralised) | - |
| 4 | Water Tank | Yes / No | - |
| 5 | Other systems | Yes / No | NA |

#### e	Rising Mains

| Dry Risers |  |  |  |
| --- | --- | --- | --- |
| S/No | Location of Inlet | Inlet No. | Floor Served |
| 1 | Staircase SE-2 landing | 5-7 | All floors |
| 2 | Staircase SE-3 landing | 6-7 | All floors |
| 3 | Staircase SE-1 landing | 7-7 | All floors |
| Wet Risers |  |  |  |
| 1 | NONE | N/A | N/A |

#### f	Fire Lifts / Staircases

| S/No | Lift No. | Staircase No. | Location | Floor served |
| --- | --- | --- | --- | --- |
| 1 | EF-2 | SE-3 | Research Wing | All floors |

#### g	Other Lifts / Staircases

| S/No | Lift No. | Staircase No. | Location | Floor served |
| --- | --- | --- | --- | --- |
| 1 | EP4 | - | Between Research Wing and Innovation Wing | Basement to 7 |
| 2 | EP5 | - | Between Research Wing and Innovation Wing | Basement to 7 |
| 3 | EP6 | - | Between Research Wing and Innovation Wing | Basement to 7 |
| 4 | - | SE-1 | Research Wing | Basement to 7 |
| 5 | - | SE-2 | Research Wing | Basement to 7 |

8. 	TIC RISKS – STATE NA IF NOT APPLICABLE

| Location | Description | Contents & Quantity | Protection System |
| --- | --- | --- | --- |
| Lab | General chemical laboratory waste including hazardous chemicals and flammable solvents | 40 liters | Sprinkler; Stored in carboy within flammables safety cabinet |
| Lab | Pathogenic biological waste | 20 liters / 1 dumpster for solid waste | Secondary containment |

9. 	MITIGATION & CONTAINMENT SYSTEMS – STATE NA IF NOT APPLICABLE

| S/No | Mitigation & Containment System | Location | Remarks |
| --- | --- | --- | --- |
| 1. | Chemical spill kit | Lab 1 |  |
| 2. | Chemical spill kit | Lab 2 |  |

10.    BIOLOGICAL AGENTS – STATE NA IF NOT APPLICABLE

| Location | Description | Contents & Quantity | Protection System |
| --- | --- | --- | --- |
| Lab | Risk groups 1 & 2 agents will be used | < 500 ml at any one time | Biological spill kit |

11.    BIOLOGICAL PROTECTION SYSTEMS – STATE NA IF NOT APPLICABLE

| S/No | Biological Protection | Location | Remarks |  |
| --- | --- | --- | --- | --- |
| 1 | Biological spill kit | Lab 2 |  |  |

- RADIOACTIVE AGENTS – STATE NA IF NOT APPLICABLE

| S/No | Radiological Protection | Location | Remarks |
| --- | --- | --- | --- |
| 1 | NA |  |  |

- RADIOLOGICAL PROTECTION SYSTEMS – STATE NA IF NOT APPLICABLE

| Location | Description | Half-Life | Protection System |
| --- | --- | --- | --- |
|  | NA |  |  |

- SENSITIVE INSTALLATIONS – STATE NA IF NOT APPLICABLE

| Location | Description | Occupancy | Usage |
| --- | --- | --- | --- |
|  | NA |  |  |

- ADJACENT BUILDINGS / INSTALLATIONS

| Name of Installation | Risk Type | PED Reference No. |
| --- | --- | --- |
| The CREATE campus houses many laboratories that use and store various hazardous materials. | Sensitive Installations Flammable Materials TIC BA RA |  |

# ANNEX E – OTHER PLANS

- EVACUATION PLAN*

The evacuation routes from the Scheduled Premise are plotted in Annex B-1. This evacuation plan shall be read in conjunction with the Emergency Response Plan published by the National Research Foundation.

- ARSON PREVENTION PLAN*

Not available

- In Place Protection Plan (IPP)

The procedures for In Place Protection have been described in Section 3.2.3.3.

# ANNEX F – ROLES AND RESPONSIBILITIES

Note: The CREATE campus at University Town, National University of Singapore houses various office and research laboratories under the National Research Foundation CREATE programme. It consists of a CREATE Tower with offices and an interlinked research laboratories building denoted as the Research, Innovation and Enterprise wings. The CREATE campus Fire Safety Manager shall be the overall in-charge for all emergency response operation for the CREATE campus.

The Company shall separately appoint the Site Controller, Site Incident Controller and Fire Warden to address emergency situations within the Scheduled Premise only.

ROLES AND RESPONSIBILITIES OF CERT

SITE CONTROLLER (SC)

The SC is a senior member of the installation management. He or she is the person who is overall in-charge of the emergency response operations in the installation and liaises with senior officials of government agencies such as SCDF, SPF, NEA, etc. Under circumstances whereby the SC is unable to leave the Scheduled Premise, he/she may appoint another suitable representative to link up with the Incident Manager at TACT HQ.

The SC shall liaise with the Building Fire Safety Manager (appointed by the National Research Foundation) and provide the necessary Company specific information required for the emergency response.

## The Key Responsibilities of the SC includes:

- Coordinate the activities of external emergency organizations and work closely with the SCDF Incident Manager at SCDF’s Tactical Headquarter (TACT HQ) during consequence management;

- Approve the release of the following information to the TACT HQ:

- Site Layout Map;

- Building plan;

- Company Emergency Response Plan;

- Company’s hazmat inventory and location of hazmat inventory;

- Overall workers population;

- Incident resources available at site;

- Provide the SCDF Incident Manager with the necessary information and decisions to any actions that concerns the Company SOPs and policies;

- Authorise the shutdown of operations in the installation;

- Authorise the release of information to the media and government agencies; and

- Assist the Site Incident Manager in determining the termination of the emergency and authorizing re-entry upon complete recovery.

SITE INCIDENT CONTROLLER (SIC)

The Company’s Site Incident Controller shall be a senior member of the installation supervisory staff. He or she is overall in-charge of the actual ground response operations for emergency occurring within the Scheduled Premise only. He / She is to provide assistance and information to SCDF Ground Commander during operation.

## The Key Responsibilities of SIC includes:

- Establishing the on-site Emergency Response Team;

- Sizing up incident situation and recommending response strategy and tactical plan;

- Determining incident control zones;

- Commanding and directing emergency response team;

- Informing the Site Controller as soon as evacuation has occurred (note that this is the case even if only the SIC has evacuated)

- Ensuring emergency responders safety and monitor personnel fatigue and stress;

- Deploying emergency equipment and appliances;

- Directing rescue operations if necessary;

- Maintaining constant communication with SC and the emergency responders;

- Working closely with SCDF ground officer;

- Ensuring proper decontamination of the equipment and responders;  and

- Coordinating recovery activities.

EMERGENCY RESPONSE TEAM  (ERT)

The ERT consists of personnel trained in basic emergency response actions such as fire fighting, HazMat mitigation and other supporting activities such as security, evacuation, first aid etc.

In the event that the emergency occurs outside the Scheduled Premise and is not caused by any activities associated with the Company operation, the ERT shall have no role in the building level ground response operation.

## The Key Responsibilities of ERT includes:

- Be familiar with the operation of the fire alarm system and use of first aid, firefighting equipment;

- Conduct basic emergency response actions such as firefighting, HazMat mitigation and termination of gas supplies under the command of SIC for emergency occurring within the Scheduled Premise only;

- Assist in emergency notification of neighboring premises and conduct public protective actions under the command of SIC;

- Assist SIC / SC to Implementing In-Place protection (IPP) within the workplace under the command of SIC;

- First Aid personnel should perform basic first aid on any casualty;

- Conduct investigation of gas leak incidents and implement recovery measures approved by the SIC and SC.

- In the event that the Lab Manger / Site Incident Controller is unavailable, the ERT shall assume those responsibilities

OTHER APPOINTMENT HOLDERS

FIRE SAFETY MANGER / ASSISTANT FIRE SAFETY MANGER

(To be appointed by the building owner)

## The Key Responsibilities of FSM / ASSISTANT FSM includes:

- Ensure at all times that fire safety requirements contained in the building’s Emergency Response Plan are complied with;

- Supervise the maintenance of all fire safety works in the building;

- Ensure at all times that the occupant load of any part of any building does not exceed the capacity prescribed under the Fire Code;

- Conduct daily checks within the building common area and remove or cause to be removed any fire hazard that is found within the common area;

- Prepare an Emergency Response Plan for the premises and conduct fire drills for the occupants annually as may be stipulated in Fire Safety (Fire Safety Managers) Regulations;

- Ensure that all occupants are familiar with the means of escape located within the premises;

- Co-ordinate and supervise the occupants within the premises in fire fighting and in evacuation in the event of fire or other emergencies;

- Supervise the operation of the Fire Command Centre in the event of fire or other emergencies;

- Notify the SCDF immediately upon the occurrence of any fire or other emergencies and fire related mass casualty incident in the premises.

FIRE WARDENS / ASSISTANT FIRE WARDENS

## The Key Responsibilities of fire wardens / ASSISTANT fire wardens includes:

- During an evacuation, the Fire Warden shall direct all staff to evacuate the facility via the nearest exit stairway;

- Check to ensure that there is no one still remaining within the Scheduled Premise before leaving;

- Ensure on the smooth and orderly evacuation of all Company employees by guiding them to designated Evacuees Assembly Area (EAA);

- Mark attendance at Assembly Area and account for all personnel inside and outside of the hazard risk zone. Report status to Building Fire Safety Manager.

- If the Lab Manager is not around or is occupied with emergency response action within the premise, the Fire Warden shall be responsible for delivering the emergency docket containing the staff name list and contact info, MSDS of all chemicals within facility, Emergency Response Plan, chemical list, facility layout plans, and all other documents deemed useful for SCDF to plan its operation. The emergency docket shall be located at a prominent location within the lab at all times.

LAB MANAGER

## The Key Responsibilities of LAB MANAGER includes:

- Represent CARES in CREATE safety committee;

- Be familiar with the Emergency Response Plan and means of escape of the building;

- Acquaint any new employee with the Emergency Response Plan including his specific role (if any) during an emergency;

- Be familiar with the operation of the fire alarm system and use of first aid, firefighting equipment.

- Ensure that all fire-fighting equipment within the Scheduled Premise is maintained.

- Maintain the emergency docket and ensure that all documents within are up-to-date;

  - Maintain updated list of Company Chemical Inventory;

  - Maintain updated list of all personnel authorised to use the lab, including their contact info for attendance checking

  - Ensure proper documentation of risk assessments of all chemicals used and experiments performed at the lab by relevant parties

  - Maintain updated facility layout plan

- Develop and implement Chemical Spill Control Measures;

- Organise chemical and gas safety and emergency response trainings for all staff; Ensure that all spill kits located within the Scheduled Premise are maintained;

- Ensure the proper maintenance of all emergency equipment within the facility;

- Ensure periodical maintenance and calibration of all gas detectors within facility;

- Ensure periodical maintenance of gas delivery system, including pressure check for coaxial pipings interstitial sheath gas and working conditions of valves in particular solenoid valves;

- Ensure periodical maintenance and check of all gas emergency call buttons.

# ANNEX G – DETAILED GROUPING AND TASKS

| S/NO | GROUPING | GENERAL TASKS | PHASE | DETAILED TASKS | REMARKS |
| --- | --- | --- | --- | --- | --- |
| 1 | Site Controller | Overall In-charge of emergency response operations and liaise with building Site Main Controller and government agencies. | I | - Carry out initial Response & call SCDF / SPF - Notify NEA and/or MOM when required - Summon ambulance if employee is injured - Notify Building Fire Safety Manager of emergency - Activate evacuation |  |
|  |  |  | II | - Authorise shutdown of equipment |  |
|  |  |  | III | - Liaise with SCDF and Building Fire Safety Manager for re-entry instruction |  |
| 2 | Site Incident Controller | Overall In-charge of actual ground response operations. | I | - Activate on-site CERT | The SIC shall be the leader of the CERT |
|  |  |  | II | - Supervise evacuation of Company personnel; Supervise on-site emergency response & coordinate operation with SCDF team |  |
|  |  |  | III | - Supervise on-site cleanup and recovery operation |  |
| 3 | Emergency Response Team | Responsible for fire fighting, rescue & first aid. | I | - Respond to CERT activation |  |
|  |  |  | II | - Conduct basic emergency response such as firefighting, Hazmat mitigation, containment & first aid. |  |
|  |  |  | III | - Perform cleanup operation |  |
| 4 | Building Fire Safety Manager | Overall In-charge of Evacuation procedure & liaise with SC | I | - Carry out initial Evacuation Procedure | Appointed by building owner |
|  |  |  | II | - Liaise with SCDF on emergency response for building |  |
|  |  |  | III | - Liaise with SCDF on recovery operation for building |  |

# ANNEX H – EMERGENCY SPILL CONTAINMENT PROTOCOLS

Emergency Spill Containment Protocol

- Don the appropriate Personal Protective Equipment (PPE) before cleaning up any chemical/biological spill. Wear disposable laboratory coat, goggles or face shield, hand gloves, rubber boots and face mask, where appropriate. Note chemical resistivity of hand glove material (neoprene gloves provided) and appropriateness of face mask to protect against respiratory hazard. Respirator shall not be used unless the user has been fit tested. Refer to MSDS for spilled chemical when in doubt.

- If the spill generates flammable vapor, take precautions to avoid spark generation such as switching on/off or unplugging of any electrical equipment nearby.

- If appliances with hot surfaces such as hot plates and heating mantle are near to the spill, turn off the main electricity supply to the benches.

- Transfer any leaking and damaged containers to oversized containment bucket for eventual disposal by licensed toxic industrial waste collector.

- Contain spill using adsorbent booms. Soak up spill using adsorbent pads. Use tongs when performing the process.

- Place all contaminated adsorbent materials in enclosed containers for eventual disposal by licensed toxic industrial waste collector. Clearly label all filled containers accordingly.

- Upon completion of the decontamination operation, clean up the affected area (using chemical disinfectant for biological spill) and thoroughly wash yourself clean of all chemical traces. Place the contaminated PPE in a plastic bag for disposal by licensed toxic industrial waste collector.

- If the scale of the spill is beyond your control, close the lab door, lab the spill area with warning signs and evacuate the lab immediately. Activate the fire alarm and describe the spill situation to security, fire warden, fire safety manager as well as SCDF ground commander at the Assembly Area upon their arrival.

- For spill of common acids, cover the contaminated areas with large excess of solid acid spill neutralizer. (Do this carefully as the process may be exothermic!) Transfer the mixture carefully in small portions into a large container of excess water. Retain the container of water for disposal by licensed toxic waste collector.

- For spill of common bases, cover the contaminated areas with large excess of solid base spill neutralizer. (Do this carefully as the process may be exothermic!) Transfer the mixture carefully in small portions into a large container of excess water. Retain the container of water for disposal by licensed toxic waste collector.

CHEMICAL SPILL KIT CONTENTS:

• 1 roll 3M Folded Chemical Sorbent
• 1 pc of DU PONT Disposable Tyvek Coverall
• 1 pc of AOSAFETY Safety Chemical Goggle
• 1 set of 3M Half Mask Respiratory w/Cartridges
• 1 pair of ANSELL Chemical Gloves
• 1 pair of WAYNE Chemical Resistant PVC Boots
• 1 Disposable Bag & Cable Ties
• 1 Plastic Container with Handle

BIOLOGIAL SPILL KIT CONTENTS:

- The contents of the chemical spill kit shall be used during a biological spill emergency too. In addition, one of the following appropriate disinfectant will be used.

- Bleach (final 10% sodium hypochlorite) / 70% (v/v) ethanol

# ANNEX I – CERT STRUCTURE

MAR

# ANNEX J – EMERGENCY RESPONSE FLOWCHART